You have to program the bin files LEFT and RIGHT in the EPROM’S If you Finished the romboard. If you not have a Eprom Programmer ask it to Electronica Specialist

The next steps are very simple:

1. disconnect your "Z" from the mains with rear switch.

2. plugin the romboard as shown in the picture

3. connect the "Z" via a RS232 Nullmodemcabel with your PC

4. open WDN40OAK and open the file +CAM_Z.HDF bautrade can High 112500 and check your com port

5. click on download -> waiting for connection

6. Press the button on the romboard to activate it and switch on the mains with the rear switch.

7. Now the "Z" begins booting with th H208 Loader and also begins downloading from the PC. At this point you must release the buttton on the romboard....

8. After the complete downloading and programming, you have to press the Standby-button. After that, you have to disconnect the - no, it is not anymore a "Z" - from the mains and remove the romboard. Thats all...

9. The +CAM_Z.HDF ist a specially build HDF, which contains a complete flashimage including settings, loaders, application, screens and the new deviceID 0016:0600. In other words the "Z" is a normal 5400 without any difference to the last build ones...

10. If you have some troubles look at document Important

have fun